8. DESIGN

8.1	GENERAL

This chapter deals with actual design calculations and detailing diagrams of the following structural components:

- Slabs

- Beams

- Columns

Design of raft foundation is beyond the scope of this project.

8.2	SLAB DESIGN

Typical design calculations for slab 6S16 are shown below:

Fig. 8.1 Slab 6S16

Center-to-center dimensions of the slabs are as given:

Lx = 3.86 m and Ly = 5.14 m

Effective depth ≈ Lx / 30 = 5140 / 30 	≈ 171 mm

(Assuming l/d ratio of 30 - to be verified later)

with clear cover of 15 mm and say, 10φ bars, overall thickness of slab

D = 171 + 15 + 10/2 = 191 mm

 Provide D = 200 mm

 dx = 200 - 15 - 5 = 180 mm

 dy = 200 - 15 -10 - 5 = 170 mm

Hence, effective spans, lx = 3860 - 230 + 180 = 3810 mm

ly = 5140 - 230 + 170 = 5080 mm

 r = lylx = 1.34

Loads on Slab

Self wt. @ 25 kN/m3 × 0.200 = 5.0 kN/m2

Finishes			= 1.5 kN/m2

Partition wall			= 1.0 kN/m2

Live load (accessible terrace) = 1.5 kN/m2

--------------

Total = 9.0 kN/m2

 Factored load = 1.5 × 9.0 = 13.5 kN/m2

Design Moments

For r = 1.34, and two long edge being continuous but two short edges being half-continuous and half-discontinuous, average value of α’s are used for moment calculation.

+Mx = 0.04×13.5×3.812 = 7.84 kNm/m

+My = 0.028×13.5×3.812 = 5.49 kNm/m

-Mx = 0.052×13.5×3.812 = 10.19 kNm/m

-My = 0.037×13.5×3.812 = 7.25 kNm/m

Now, Rux=Mxbdx2and similarly for Ruy

 Rx+ = 0.24  pt = 0.12% (from SP:16 design aids)

 Rx- = 0.31  pt = 0.12%

 Ry+ = 0.19  pt = 0.12%

 Ry- = 0.25  pt = 0.12%

Note that minimum steel governs in all cases. Thus thickness provided is more than adequate.

Requisite steel, Ast along x direction = 216 mm2  8φ-230c/c ≤ 300 mm. OK

along y direction = 204 mm2  8φ-240c/c ≤ 300 mm. OK

Adopt 8φ-230c/c as main and distributor reinforcement in both X and Y direction.

Deflection check

Area of cross-section of steel required = 216 mm2

Area of cross-section of steel provided = 218.5 mm2

pt provided = 0.12%

fs=0.58×415×216218.5 = 238

Hence, multiplier factor for tension reinforcement (from Fig.4 of IS 456:2000) > 1.5

Thus permissible l/d > 1.5 × 23 = 34.5

l/d provided = 30 < 34.5. Hence deflection is under control. OK

Torsion Reinforcement

Area of full torsion reinforcement = 3/4th of 216 = 54 mm2 per meter

Area of half torsion reinforcement = 3/8th of 216 = 27 mm2 per meter

 2 meshes to be provided at top and bottom each with spacing 300 mm c/c.

Reinforcement detailing is shown in Fig. 8.2

Design moments and reinforcements spacing of all other slabs are tabulated in Appendix B. Due to space constraint only one set of design calculation and only few slabs detailing is presented in this report.

8.3	BEAMS

Due to time and space constraint, design calculations and detailing diagrams of three beams, namely B7, B16 and B23 forming a frame (YZ plane at X = 1.34 m and 
19.13 m) only are presented.

8.3.1	Design Load Effects

Governing load combination for these three beams as obtained from SAP2000 analysis is produced in Table 8.1.

Table 8.1 Governing load combination for B7, B16 and B23

| Floor |  | B7 |  | B16 |  | B23 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | V22 (kN) | M33 (kNm) | V22 (kN) | M33 (kNm) | V22 (kN) | M33 (kNm) |
| 0 | Min | -50 | -61 | -45 | -41 | -56 | -41 |
|  | Max | 103 | 42 | 40 | 31 | 59 | 37 |
| 1 | Min | -46 | -58 | -41 | -34 | -59 | -51 |
|  | Max | 102 | 35 | 36 | 24 | 60 | 39 |
| 2 | Min | -41 | -56 | -40 | -32 | -44 | -31 |
|  | Max | 104 | 32 | 34 | 21 | 51 | 26 |
| 3 | Min | -38 | -56 | -38 | -30 | -41 | -28 |
|  | Max | 106 | 29 | 33 | 19 | 49 | 23 |
| Floor |  | B7 |  | B16 |  | B23 |  |
|  |  | V22 (kN) | M33 (kNm) | V22 (kN) |  | V22 (kN) | M33 (kNm) |
| 4 | Min | -35 | -57 | -37 | -28 | -37 | -25 |
|  | Max | 107 | 30 | 31 | 16 | 47 | 20 |
| 5 | Min | -33 | -58 | -36 | -26 | -36 | -24 |
|  | Max | 108 | 23 | 29 | 13 | 48 | 17 |
| 6 | Min | -13 | -33 | -9 | -13 | -14 | -12 |
|  | Max | 57 | 14 | 3 | 6 | 24 | 11 |

Since load effects are likely to act in opposite direction also due to reversibility of loadings (Refer BMDs and SFDs in Chapter 6), sectioned is designed such that it can resist the force / moment in both directions. As we can observe, there is very little variation in load effects in beams across the floors except on 6th floor, it is practical to design the sections for maximum of these values and provide same reinforcement at all floor levels of corresponding beam. Because of aforementioned two considerations, design values of the load effects are taken as follows:

Table 8.2 Design load effects for B7, B16 and B23

| Design load effects | B7 |  | B16 |  | B23 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | V22 (kN) | M33 (kNm) | V22 (kN) | M33 (kNm) | V22 (kN) | M33 (kNm) |
| Min | 108 | -61 | 40 | -41 | 60 | -51 |
| Max |  | 42 |  | 31 |  | 37 |

Following the codal provisions as specified in the Chapter 7, design calculations are presented below.

8.3.2	Design of B7

All section size: 200 mm × 350 mm

Assuming bar diameter of 20 mm, effective depth d = 350 - 30 (clear cover) - 20/2 = 310 mm

M33+bd2 = 42×106200×3102 = 2.19  pt = 0.688% 	(From SP:16 design aids)

M33−bd2 = 61×106200×3102= 3.17  pt= 1.071%

Since bothpt ≥ 0.29% minimum steel and ≤ 2.5% maximum steel. OK

Ast, joint = 664 mm2  provide 3 bars of 18 mm φ => 763 mm2

Ast,midspan = 427 mm2  provide 2 bars of 18 mm φ => 509 mm2

Positive steel = 509 > ½ (Negative steel = 763)	OK

Top and bottom steel at any section = 509 > ¼ ×(Negative steel at joint = 763)	OK

Design Shear

The ultimate moment of resistance of the section at joint, where Ast = Asc = 763 mm2, is limited to

Mu,limh=0.362fckbxu,max(d−0.416xu,max)+(fsc−0.447fck)Asc(d−d')

For xu,max= 0.48 × 311 = 149 mm and fsc at xu,max = 347.15 MPa for d'd = 0.125;

Mu,limh = 67.16 + 69.8 = 136.96 kNm

Also, UDLDL = 28.4 kN/m and UDLLL = 4.3 kN/m

Vu,b=VbD+L+1.4Mu,limAs+Mu,limBhLAB

Vu=1.2(28.4+4.3)3.842+1.4136.96+136.963.84 = 175.1 kN > 108 kN

Hence design shear stress, τu=175.1×103200×311 = 2.82 ≤ 3.1 maximum permissible shear stress for M25 concrete. OK

τc = 0.57 for M25 concrete with 0.80%

Thus, design shear force to be carried by steel hoops is Vus=(τu−τc)bd

 Vus = 140 kN

For 8φ bars as stirrups, Asv = 100.5 mm2

Spacing = 0.87fyAsvVus = 80 mm

Maximum spacing = minimum of (0.75d, 300,Asvfy0.4b, d/2)

= minimum of (233, 300, 520, 155) = 155 mm > 80 mm provided.

Near beam column junction, spacing = minimum of (d/4, 8×18) = 77.5 mm

Adopt 8φ - 80 mm c/c for whole beam.

Development length in tension = Ldt = 0.87fyφ4τd = 0.87×415×184×1.4×1.6 = 725 mm

8.3.3	Design of B16

M33+bd2 = 31×106200×3102 = 1.60  pt = 0.482% 	(From SP:16 design aids)

M33−bd2 = 41×106200×3102= 2.12  pt= 0.659%

Since bothpt ≥ 0.29% minimum steel and ≤ 2.5% maximum steel. OK

Ast, joint = 410 mm2  provide 2 bars of 18 mm φ => 509 mm2

Ast,midspan = 300 mm2  provide 2 bars of 18 mm φ => 509 mm2

Positive steel = 509 > ½ (Negative steel = 509)	OK

Top and bottom steel at any section = 509 > ¼ ×(Negative steel at joint = 509)	OK

Design Shear

The ultimate moment of resistance of the section at joint, for Ast = Asc = 509 mm2,

xu,max= 0.48 × 311 = 149 mm and fsc at xu,max = 347.15 MPa for d'd = 0.125;

Mu,limh = 67.16 + 45.6 = 112.76 kNm

Also, UDLDL = 15.24 kN/m and UDLLL = 1.02 kN/m

Vu=1.2(15.24+1.02)3.282+1.4112.76×23.28 = 128.3 kN > 40 kN

Hence design shear stress, τu=128.3×103200×311 = 2.07 ≤ 3.1;		OK

τc = 0.59 for M25 concrete with 0.82%

 Vus = 140 kN

For 8φ bars as stirrups, spacing = 0.87fyAsvVus = 120 mm

Maximum spacing = minimum of (233, 300, 520, 155) = 155 mm > 120 mm provided.

Near beam column junction, spacing = minimum of (d/4, 8×18) = 77.5 mm

Adopt 8φ - 120 mm c/c for whole beam, except 650 mm from each junction for which use 8φ - 100 mm c/c.

8.3.4	Design of B23

M33+bd2 = 51×106200×3102 = 2.64  pt = 0.852%

M33−bd2 = 37×106200×3102= 1.91  pt= 1.586%

Since bothpt ≥ 0.29% minimum steel and ≤ 2.5% maximum steel. OK

Ast, joint = 365 mm2  provide 2 bars of 18 mm φ => 509 mm2

Ast,midspan = 530 mm2  provide 3 bars of 18 mm φ => 763 mm2

Positive steel = 509 > ½ (Negative steel = 763)	OK

Top and bottom steel at any section = 509 > ¼ ×(Negative steel at joint = 763)	OK

Design Shear

The ultimate moment of resistance of the section at joint, for Ast = Asc = 509 mm2, xu,max= 0.48 × 311 = 149 mm and fsc at xu,max = 347.15 MPa for d'd = 0.125;

Mu,limh = 67.16 + 45.6 = 112.76 kNm

Also, UDLDL = 15.9 kN/m and UDLLL = 2.1 kN/m

Vu=1.2(15.9+2.1)3.472+1.4112.76×23.47 = 129.2 kN > 60 kN

Hence design shear stress, τu=129.2×103200×311 = 2.08 ≤ 3.1 		OK

τc = 0.59 for M25 concrete with 0.82%

 Vus = 92.7 kN

For 8φ bars as stirrups, spacing  = 120 mm

Maximum spacing = minimum of (233, 300, 520, 155) = 155 mm > 120 mm provided.

Near beam column junction, spacing = minimum of (d/4, 8×18) = 77.5 mm

Adopt 8φ - 120 mm c/c for whole beam, except 650 mm from each junction for which use 8φ - 100 mm c/c.

Detailing diagrams of these frames are shown in Fig. 8.3 after section 8.4

8.4	COLUMN DESIGN

Total number of columns groups in the building is 11 i.e. there are 11 sizes of columns in the building. The design is based on maximum load effects for each size and similar reinforcement is proposed in all columns of that size. Also design for each column group is performed for load effects at ground floor and same reinforcement is continued till 2nd or 3rd floor. For columns above that new reinforcement detailing is calculated, which will continue till top of the building. Due to strength requirement first three floor columns, Ground floor, First floor and second floor columns, have concrete of grade M35 while columns above that have M25 concrete.

8.4.1	Design Load Effects

Following are design loads for which each column group is designed. Due to reversibility of moments, longitudinal reinforcement is equally distributed on all four sides. Hence absolute maximum values of load effects are taken for design.

Table 8.3 Design loads for columns

| Column size (mm × mm) | Floor level | P (kN) | M22 (kNm) | M33 (kNm) |
| --- | --- | --- | --- | --- |
| 250 × 250 | 0 | 1235.0 | 24.0 | 17.0 |
|  | 3 | 638.7 | 21.2 | 13.0 |
| 250 × 300 | 0 | 1250.2 | 27.4 | 39.5 |
|  | 3 | 693.2 | 38.8 | 21.4 |
| Column size (mm × mm) | Floor loading | P (kN) | M22 (kNm) | M33 (kNm) |
| 250 × 400 | 0 | 2361.3 | 37.0 | 88.8 |
|  | 4 | 839.7 | 34.0 | 51.9 |
| 300 × 300 | 0 | 1439.8 | 45.5 | 41.8 |
|  | 3 | 812.6 | 40.5 | 28.1 |
| 300 × 350 | 0 | 1478.6 | 53.3 | 60.1 |
|  | 3 | 814.7 | 32.4 | 39.8 |
| 300 × 400 | 0 | 1949.7 | 82.7 | 91.3 |
|  | 4 | 765.1 | 30.6 | 42.7 |
| 300 × 500 | 0 | 2561.1 | 77.0 | 174.8 |
|  | 4 | 1054.5 | 44.3 | 96.5 |
| 350 × 350 | 0 | 2031.7 | 77.5 | 71.8 |
|  | 4 | 911.5 | 29.2 | 28.7 |
| 400 × 400 | 0 | 2142.5 | 117.9 | 106.0 |
|  | 4 | 1163.4 | 42.2 | 37.9 |
| 400 × 450 | 0 | 2555.2 | 142.5 | 171.0 |
|  | 3 | 1415.9 | 105.0 | 38.5 |
| 500 × 500 | 0 | 3404.8 | 267.6 | 267.1 |
|  | 3 | 1946.8 | 75.0 | 59.4 |

Design calculations of few columns are presented below as examples. Complete design details of all column groups is tabulated in Table 8.4. Cross-Sectional diagrams of all column groups are shown in Appendix C.

8.4.2	Design of Column group 250 mm × 250 mm : Ground floor

Pu = 1235 kN

M22 = 24 kNm  e22 = 24×1031235 = 19.4 mm < 20 mm, minimum eccentricity

M33 = 17 kNm  e33< 20 mm

Permissible eccentricity, 0.05D = 0.05×250 = 12.5 mm < 20 mm

Hence column is to be designed for minimum biaxial moment,

Mdesign, 22 = Mdesign, 33 = 1235×20×10-3 = 24.7 kNm

Now, PufckbD=1235×10335×2502 = 0.56

For finding initial trial percentage of steel, define equivalent moment Meq=1.2Mx2+My2

 MeqfckbD2=0.08

For 40 mm clear cover and 10 mm φ bars, d’ = 40+20/2 = 50

d’ / D = 50 / 250 = 0.20

From design aids, for M35 and Fe415 corresponding to d’/D = 0.20 and picking value of ptfck for ordinate 0.56 and abscissa 0.08,

pt = 0.12×35 = 4.2%

Trying for pt = 4.5%

Puz=0.45fckAc+0.75fyAsc

Puz=0.45×35×(1−0.045)+0.75×415×0.045250×250= 1815.5 kN

PuPuz=12351815.5=0.68

Hence α=0.68−0.20.6+1.0 = 1.80

For PufckbD = 0.56 and ptfck = 0.13, Mu1fckbD2 turns out to be 0.09

 Mux and Muy = 0.09×35×250×2502 = 49.2 kNm

Hence the value of the expression

MuxMuxα+MuyMuyα = 24.749.21.80+24.749.21.80 = 0.58 ≤ 1.0	OK

Area of steel corresponding to pt = 4.5%  2813 mm2  12 bars of 18 mm φ => 3048 mm2

Lateral Ties

Use 16φ at spacing of minimum of (least lateral dimension, 16× φ, 300mm, ½ of least lateral dimension)

 minimum of (250, 16×18, 300, 250/2) = 125

Adopt 16φ - 125 mm c/c

Special Confining Reinforcement

Spacing = 250 / 4 = 62.5 mm, but ≥ 75 mm and ≤ 100 mm

 adopt 16φ - 75 mm c/c

Area of steel required =0.18ShfckfyAgAk−1.0,

= 0.18×75×202×3541525022022−1.0 = 122 < 201 mm2 provided by 16φ. OK

8.4.3	Design of Column group 250 mm × 400 mm : Fourth floor

Pu = 840 kN

M22 = 34 kNm  e22 = 34×103840 = 40.5 mm > 20 mm

M33 = 51.8 kNm  e33> 20 mm

Permissible eccentricity, 0.05D = 0.05×250 = 12.5 mm < 20 mm

Hence column is to be designed for biaxial moments.

PufckbD=840×10325×250×400 = 0.34

MeqfckbD2 = 1.2(342+51.82)×10625×400×2502=0.12

d’ / D22 = 50 / 250 = 0.20

d’ / D33 = 50 / 400 = 0.12 (using avg. value of design chart corresponding to d’/D = 0.10 and d’/D = 1.15)

pt = 0.12×25 = 3%

Trying for pt = 3%

Puz=0.45×25×(1−0.03)+0.75×415×0.03250×400= 2025 kN

PuPuz=8402025=0.41

Hence α=0.41−0.20.6+1.0 = 1.36

For PufckbD = 0.34 and ptfck = 0.12,

 M22= 0.12×255×400×2502 = 75 kNm

 M33 = 0.145×25×250×4002 = 145 kNm

Hence the value of the expression

34751.36+51.81451.36 = 0.59 ≤ 1.0	OK

Area of steel corresponding to pt = 3%  3000 mm2  12 bars of 18 mm φ => 3048 mm2

Lateral Ties

Adopt 16φ - 125 mm c/c

Special Confining Reinforcement

 adopt 16φ - 75 mm c/c

Area of steel required =0.18×75×352×25415250×400202×352−1.0 = 163 < 201 mm2 provided by 16φ. OK

8.4.4	Design of Column group 300 mm × 500 mm : Ground floor

Pu = 2561 kN

M22 = 77 kNm  e22 = 77×1032561 = 30 mm > 20 mm

M33 = 174.8 kNm  e33> 20 mm

Permissible eccentricity, 0.05D = 0.05×300 = 15 mm < 20 mm

Hence column is to be designed for biaxial moments.

PufckbD=2561×10335×300×500 = 0.49

MeqfckbD2=0.14

For 40 mm clear cover and 10 mm φ bars, d’ = 40+20/2 = 50

d’ / D2 = 50 / 300 = 0.16

d’ / D3 = 50 / 500 = 0.10

pt = 0.14×35 = 4.9%

Trying for pt = 3.5%

Puz=0.45×35×(1−0.035)+0.75×415×0.035300×500= 3914 kN

PuPuz=25613914=0.65

Hence α=0.65−0.20.6+1.0 = 1.76

 M22= 0.09×35×500×3002 = 142 kNm

 M33 = 0.095×35×300×5002 = 249.4 kNm

Hence the value of the expression

771421.76+174.8275.61.76 = 0.87 ≤ 1.0	OK

Area of steel corresponding to pt = 3.5%  5250 mm2  20 bars of 18 mm φ => 5089 mm2 ≈ 5250 mm2

Lateral Ties

Adopt 16φ - 150 mm c/c

Special Confining Reinforcement

Spacing = 300 / 4 = 75 mm, but ≥ 75 mm and ≤ 100 mm

 adopt 16φ - 75 mm c/c

Area of steel required = 0.18×75×452×35415300×500252×452−1.0 = 163 < 201 mm2 provided by 16φ. OK

8.4.5	Design of Column group 300 mm × 500 mm : Fourth floor

Pu = 1054.5 kN

M22 = 44.3 kNm  e22 = 44.3×1031054.5 = 42 mm > 20 mm

M33 = 96.5 kNm  e33> 20 mm

Now, PufckbD=1054.5×10325×300×500 = 0.28

MeqfckbD2=0.11

pt = 0.07×25 = 1.75%

Trying for pt = 1%

Puz=0.45×25×(1−0.01)+0.75×415×0.01300×500= 2137.5 kN

PuPuz=1054.52137.5=0.49

Hence α=0.49−0.20.6+1.0 = 1.49

 M22 = 0.08×25×500×3002 = 90 kNm

 M33 = 0.085×25×300×5002 = 159.4 kNm

Hence the value of the expression

44.3901.49+96.5159.41.49 = 0.82 ≤ 1.0	OK

Area of steel corresponding to pt = 1%  1500 mm2  8 bars of 18 mm φ => 2032 mm2

Lateral Ties

Adopt 16φ - 125 mm c/c

Special Confining Reinforcement

 adopt 16φ - 75 mm c/c

8.4.6	Design Information of all Columns groups

Detailed reinforcement diagram of columns C4, C7, C10 and C13 is presented in
Fig. 8.3(a) and (b). Detailing of remaining columns is not shown in the report because of space constraint. Table 8.4 summarises design information of all column groups.

Table 8.4 Reinforcement details of columns

| Column Group | Floors | pt(%) | # Bars | Bar dia. (mm) | hoop spacing (mm) | SCR* spacing (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| 250 × 250 | 0,1,2 | 4.5 | 12 | 18 | 125 | 75 |
| 250 × 250 | 3,4,5,6 | 3.0 | 8 | 18 | 125 | 75 |
| 250 × 300 | 0,1,2 | 4.0 | 12 | 18 | 125 | 75 |
| 250 × 300 | 3,4,5,6 | 2.7 | 8 | 18 | 125 | 75 |
| 250 × 400 | 0,1,2,3 | 5.25 | 20 | 18 | 125 | 75 |
| 250 × 400 | 4,5,6,7,8 | 3.0 | 12 | 18 | 125 | 75 |
| 300 × 300 | 0,1,2 | 3.0 | 12 | 18 | 150 | 75 |
| Column Group | Floors | pt(%) | # Bars | Bar dia. (mm) | hoop spacing (mm) | SCR* spacing (mm) |
| 300 × 300 | 3,4,5,6 | 1.5 | 8 | 18 | 150 | 75 |
| 300 × 350 | 0,1,2 | 3.9 | 12 | 18 | 150 | 75 |
| 300 × 350 | 3,4,5,6 | 1.9 | 8 | 18 | 150 | 75 |
| 300 × 400 | 0,1,2,3 | 3.5 | 16 | 18 | 150 | 75 |
| 300 × 400 | 4,5,6,7,8 | 0.8 | 4 | 18 | 150 | 75 |
| 300 × 500 | 0,1,2,3 | 3.5 | 20 | 18 | 150 | 75 |
| 300 × 500 | 4,5,6,7,8 | 1.0 | 8 | 18 | 150 | 75 |
| 350 × 350 | 0,1,2,3 | 2.25 | 12 | 18 | 175 | 80 |
| 350 × 350 | 4,5,6,7,8 | 0.8 | 4 | 18 | 175 | 80 |
| 400 × 400 | 0,1,2,3 | 1.9 | 12 | 18 | 200 | 100 |
| 400 × 400 | 4,5,6,7 | 0.8 | 8 | 18 | 200 | 100 |
| 400 × 450 | 0,1,2 | 2.8 | 20 | 18 | 200 | 100 |
| 400 × 450 | 3,4,5,6 | 0.8 | 8 | 18 | 200 | 100 |
| 500 × 500 | 0,1,2 | 2.8 | 16 4 | 22 18 | 250 | 100 |
| 500 × 500 | 3,4,5,6 | 0.8 | 4 4 | 22 18 | 250 | 100 |

* Special Confining Reinforcement

NOTE-1: Grade of concrete in columns on ground floor and 1st and 2nd floor is M35, while the grade of concrete in columns on 4th and above floors is M25. Reinforcement details are as specified in the table above for the floor level mentioned along with.

NOTE-2: All stirrups are of diameter 16 mm.

8.5	SUMMARY

This chapter covers complete structural design and detailing of all the columns and slabs along with few beams. Typical design calculations are shown for each slab, beam and column. Design information in form of reinforcement requirement is provided for all slabs and columns. Detailing diagram of a frame is also appended showing reinforcement details in beam, column and at beam-column junction.